SERIAL PORT DESCRIPTION

PC serial ports are DTE's terminated in a DB25P (25-pin male) connector.

They provide full modem control and line status signals.  The serial port

signals are described below.

Pin#	Signal	Designation	Description		Direction (from PC)

---------------------------------------------------------------------------

1	GND	AA		Frame Ground		Common

2	TD	BA		Transmit Data		Out

3	RD	BB		Receive Data		In

4	RTS	CA		Request To Send		Out

5	CTS	CB		Clear To Send		In

6	DSR	CC		Data Set Ready		In

7	SG	AB		Signal Ground		Common

8	DCD	CF		Data Carrier Detect	In

20	DTR	CD		Data Terminal Ready	Out

22	RI	CE		Ring Indicator		In