NETWORK WIZARDS WIZCONSOLE EXAMPLE CONFIGURATION FILE

The following is an example WizConsole configuration file.  It is

configured for eight serial host lines (1-8), 2 additional serial lines

(9-10), and 1 parallel port (11) with a printer.  Host line 1 is logged to

serial line 9.  Host lines 2 and 3 are logged to the printer (11).  Host line

4 has output timestamping enabled.  Host lines 6 thru 8 are logged to a file.

; WizConsole configuration file

;   Comments start with a semicolon and go to end of line.

;Main Parameters

cursor underscore	       ;block or underscore

input-buffer-size 16           ;Kbytes (2-32)

log-file directory C:\wizcon output-buffer-size 4 stamp 221

screen-blank 15		       ;minutes

video auto                     ;auto, mono, color, bw

;define interfaces

interface serial four 1 5 00 2bf 2a0 2a8 2b0 2b8  ;ports 1-4

interface serial four 5 7 00 1bf 1a0 1a8 1b0 1b8  ;ports 5-8

interface serial standard 9 4 01 3f8              ;COM1

interface serial standard 10 3 01 2f8             ;COM2

interface parallel standard 11 378                ;LPT1

;interface parallel standard 11 3bc               ;LPT1 on monochrome card

;defaults for host lines (1 thru 8)

line 1 8

output-buffer-size 1           ;Kbytes

speed 9600                     ;110,300,1200,2400,4800,9600,19200,38400,57600

bits-per-character 7           ;7, 8

parity even                    ;none, odd, even, mark, space

stop-bits 1                    ;1, 2

backspace-sends delete	       ;backspace or delete

flow-control none              ;none, harware, software (in, out)

monitor-dsr on	               ;on, off

new-line-mode off	       ;on, off

output-notify beep underscore  ;beep and/or underscore, or none

scroll-memory-lines 350        ;lines (minimum of 24)

stamp 000		       ;no stamps

terminal vt220		       ;dumb, monitor, vt52, vt100, vt220, wyse50

tabs 9 17 25 33 41 49 57 65 73 ;initial tab stops

wrap on                        ;on, off

;now set parameter specific to each line

line 1

name pluto

line 2

name zep

terminal vt52

flow-control hardware

line 3

name sun

output-notify none

line 4

name gateway

terminal dumb

stamp 310

line 5

name fs1

flow-control software

line 6

name server

backspace-sends backspace

log-to-file

line 7

name cookie

bits-per-character 8

log-to-file

parity none

line 8

name slowpoke

log-to-file

;remote dialup line

line 9

remote

password foobar

speed 2400

;serial printer on this line

line 10

speed 2400

monitor-line 1

line 11

name Epson

printer

monitor-line 2 3

setup-string "\033x\000\033M"   ;draft mode, 12cpi

stamp 220			;timestamp the output

output-buffer-size 4		;Kbytes